# Project Design Document

### 1. Design Intent

- We set out to create a very fun and lighthearted experience with this project, create a fun project with ourselves to test our limits as people who had never touched electronics before this module began, to being a cute pet for people to look at and get attached to.

- It’s meant to act almost like a first prototype for a robotic pet for a kid, complete with animations and music aiming to stimulate.

### 2. Inspiration & References

- Inspired by Rocky from Project Hail Mary, R2D2 from Star Wars, The Graduation Bear by Kanye West

### 3. User Journey

Step-by-step experience of the user:

- A user walks by this robot, standing on its stage, with a tablet beside it explaining what it is, what is does.

- Intrigued, the user decides to flip the switch behind the bot’s head.

- Suddenly, it comes to life, with a fun robotic voice and an animation making it seem friendly, it introduces itself, YeezyBot.

- It goes through it’s routine, dances to a song, idles to a Mario Theme, and finally, it walks towards you, wanting to be your friend :)

### 4. Success Criteria

Define measurable success:

- Speaker and Animations startup in <1 second of each other

- User experience relies on the user only turning a switch on, and not having to interfere with any of the servo positions

- Animation goes through the entire 2:35 minute routine uninterrupted

- The robot doesn’t tip over or walk off its stage

### 5. System Overview

Yeezybot is an ESP32-based quadruped that performs a walking/dance routine with on-board OLED animation and audio playback. The ESP32 runs MicroPython and drives three peripheral subsystems . An I2C link (SCL=22, SDA=21, 400 kHz) talks to a PCA9685 16-channel PWM controller, which generates 50 Hz servo pulses for the eight leg servos — four hip joints and four knee joints arranged as a diagonal-trot gait with rear-biased back-lean to compensate for the front-heavy chassis. A dedicated SPI bus (SCK=18, MOSI=23, DC=2, RST=4, CS=5, 20 MHz) streams a 1,210-frame MONO_VLSB animation (⁠ comp4.bin ⁠) to a 128×64 SSD1309 OLED, with frame advances interleaved into the servo step loop so playback never stalls the walk. A DFPlayer Mini, driven over UART with its serial command protocol, plays the matching audio track from microSD in lockstep with the dance routine.

### 6. High-Level Block Diagram

👉 Visual diagram of system components and flow

### 7. Input / Output Mapping

| Input | Process | Output |
| --- | --- | --- |
| 7.4V power supply | Buck convertor | 5v power |
| Servo channels to pca9685 channels | Servo driver/i2c | Individual control over servos |
| esp32 | Pca9685 through SCL AND SDA | Control over channels on the driver |
| Micro SD Card | DF Player Mini | .mp3 files played over speaker |
| Imag sequences as bin files | SPI | OLED Display |

### 8. Sketches & Concepts

### 9. Final System Architecture (Labelled)