# Fabrication and Integration Process

### 1. Material Choice Justification :

Chose PLA for 3D printing for access to multiple colours and easier to manipulate post-print. Much lighter weight than resin, making it easier on the motors.

Chose MDF for the face plate to be able to easily laser cut and get precise dimensions quickly leaving room for iterations.

The parts were snap fit attached according to the movement of the mg90 servos and held together using m3 screws. The parts were sanded and snug but fluid movement was tweaked and refined.

Chose Extruded Polystyrene (XPS) Foam / High-Density Polyurethane (PU/PUR) Foam for the environment mainly for its shapeability and durability, along with its ability to hold the spray paint colours well.

2. Fabrication Steps :

1: 3D-Printing test parts for the servo motor housing

2: 3D-Printing the final servo motor housing

3: 3D-Printing the head of the bot

4: Laser-Cutting the face plate

5: Cut up XPS/PU/PUR Foam pieces into rocks and a globe stage for the environment and tablet stand.

DETAILED OVERVIEW OF THE HOUSING UNIT ASSEMBLY:

ISSUES FACED WHILE DOING THIS:

- STRUGGLE TO FIND THE EXACT SCREWS WE NEEDED

- THE PARTS WERE A VERY SNUG FIT, NO ROOM FOR SLACK

- THE SUPPORTS REQUIRED SANDING AND AFFECTED QUALITY OF PRINTS